Identification_Information:

Citation:

Citation_Information:

Originator:

Canada Centre for Remote Sensing (CCRS), Earth Sciences Sector, Natural Resources

Canada

Originator:

Comisión Nacional para el Conocimiento y Uso de la Biodiversidad (CONABIO)

Originator: Comisión Nacional Forestal (CONAFOR)

Originator: Insituto Nacional de Estadística y Geografía (INEGI)

Originator: U.S. Geological Survey (USGS)

Publication_Date: 2013

Title: 2005 Land Cover of North America at 250 meters

Edition: 3.0

Geospatial_Data_Presentation_Form: Raster digital data

Publication_Information:

Publication_Place: Ottawa, Ontario, Canada

Publisher: Canada Centre for Remote Sensing (CCRS)

Publication_Information:

Publication_Place: Sioux Falls, South Dakota, USA

Publisher: U.S. Geological Survey

Publication_Information:

Publication_Place: Mexico City, Mexico

Publisher: INEGI

Publication_Information:

Publication_Place: Montréal, Québec, Canada

Publisher: Commission for Environmental Cooperation

Online_Linkage: http://www.cec.org/naatlas/

Description:

Abstract:

This data set replaces the 2010 edition (Edition 1.0) of the 2005 Land Cover of

North America.  Following the release of the first 2005 land cover data, several

errors were identified in the data, including both errors in labeling and

misinterpretation of thematic classes. To correct the labeling errors, each

country focused on its national territory and corrected the errors which it

considered most critical or misleading. For the continental data sets (including

surrounding water fringe) 17440830 pixels (4.33% of the area) changed in the

update. The following national counts exclude the water fringe: Canada, 10223412

pixels changed (6.44%); Mexico, 141142 pixels changed (0.45%), and U.S., 6878656

pixels changed (4.54%).

The countries worked together to produce a definitive list of land cover

classifications for the 2005 data; this document is available for download from the

same site as the data and is entitled: North American Land Cover Classifications

(2005).

Version 1 of the 2005 North American Land Cover data set was produced as part of

the North American Land Change Monitoring System (NALCMS), a trilateral effort

between the Canada Centre for Remote Sensing, the United States Geological Survey,

and three Mexican organizations including the National Institute of Statistics and

Geography (Instituto Nacional de Estadistica y Geografia), National Commission for

the Knowledge and Use of the Biodiversity (Comisión Nacional Para el Conocimiento y

Uso de la Biodiversidad) and the National Forestry Commission of Mexico (Comisión

Nacional Forestal).  The collaboration is facilitated by the Commission for

Environmental Cooperation, an international organization created by the Canada,

Mexico, and United States governments under the North American Agreement on

Environmental Cooperation to promote environmental collaboration between the three

countries.

The general objective of NALCMS is to devise, through collective effort, a

harmonized multi-scale land cover monitoring approach which ensures high accuracy

and consistency in monitoring land cover changes at the North American scale and

which meets each country’s specific requirements. The data set of 2005 Land Cover

of North America at a resolution of 250 meters is the first step toward this goal.

The initial data set used to generate land cover information over North America was

produced by the Canada Centre for Remote Sensing from observations acquired by the

Moderate Resolution Imaging Spectroradiometer (MODIS/Terra).  All seven land

spectral bands were processed from Level 1 granules into top-of-atmosphere

reflectance covering North America at a 250-meter spatial and 10-day temporal

resolution.

In order to generate a seamless and consistent land cover map of North America,

national maps were generated for Canada by the CCRS; for Mexico by INEGI, CONABIO,

and CONAFOR; and for the United States by the USGS.  Each country used specific

training data and land cover mapping methodologies to create national data sets.

This North America data set was produced by combining the national land cover

data sets.

This version 3.0 of the North America Land Cover map at 250 meters spatial resolution

has removed the water buffer along the coastline of North America, this will ensure

consistency in the statistics calculations of class 18 (water) without incorporating

ocean water.

Purpose:

Information on land cover across North America provided by this data set is

valuable for a range of audiences, including international organizations such as

the United Nations Environment Programme, nongovernmental conservation

organizations, land managers, and scientific researchers.  The continental scale

land cover data generated under NALCMS can be used to address issues such as

climate change, carbon sequestration, biodiversity loss, and changes in ecosystem

structure and function, by helping users to better understand the dynamics and

continental-scale patterns of North America’s changing environment.

The North American Atlas data are intended for geographic display and analysis at

the national and continental level.  These data should be displayed and analyzed at

scales appropriate for 1:10,000,000-scale data.  No responsibility is assumed by

Natural Resources Canada, Instituto Nacional de Estadística y Geografía, the U.S.

Geological Survey, or the Commission for Environmental Cooperation in the use of

these data.

Supplemental_Information:

The standard distribution formats for this data set are listed below.  Users

interested in receiving the data as a GRID or BIN file may contact the CEC.

For more information on NALCMS, see http://www.cec.org/Page.asp?PageID=122&ContentID=25501.

Natural Resources Canada has North American Land Cover information available online at

http://www.nrcan.gc.ca/earth-sciences/geography-boundary/remote-sensing/optical/2208#_North_American_Land.

The U.S. Geological Survey has North American Land Cover information available online at

http://landcover.usgs.gov/nalcms.php.

For more information on MODIS, see http://modis.gsfc.nasa.gov/

The following table describes the display of land cover classification in the

.img file:

>Value  Class                                     RGB values

>----------------------------------------------------------------------------

>1      Temperate or sub-polar needleleaf forest  0     0.24  0

>2      Sub-polar taiga needleleaf forest         0.58  0.61  0.44

>3      Tropical or sub-tropical broadleaf

>         evergreen forest                        0     0.39  0

>4      Tropical or sub-tropical broadleaf

>         deciduous forest                        0.12  0.67  0.02

>5      Temperate or sub-polar broadleaf

>         deciduous forest                        0.08  0.55  0.24

>6      Mixed forest                              0.36  0.46  0.17

>7      Tropical or sub-tropical shrubland        0.7   0.62  0.18

>8      Temperate or sub-polar shrubland          0.7   0.54  0.2

>9      Tropical or sub-tropical grassland        0.91  0.86  0.37

>10     Temperate or sub-polar grassland          0.88  0.81  0.54

>11     Sub-polar or polar shrubland-lichen-moss  0.61  0.46  0.33

>12     Sub-polar or polar grassland-lichen-moss  0.73  0.83  0.56

>13     Sub-polar or polar barren-lichen-moss     0.25  0.54  0.45

>14     Wetland                                   0.42  0.64  0.54

>15     Cropland                                  0.9   0.68  0.4

>16     Barren lands                              0.66  0.67  0.68

>17     Urban                                     0.86  0.13  0.15

>18     Water                                     0.3   0.44  0.64

>19     Snow and Ice                              1     0.98  1

>

The Commission for Environmental Cooperation (CEC) is an international organization

created by Canada, Mexico, and the United States of America under the North

American Agreement on Environmental Cooperation (NAAEC).  The CEC was established

to address regional environmental concerns, help prevent potential trade and

environmental conflicts, and to promote the effective enforcement of environmental

law.  The Agreement complements the environmental provisions of the North American

Free Trade Agreement (NAFTA).  Further information on the CEC is available from

http://www.cec.org/ or from

>Commission for Environmental Cooperation

>393, rue St-Jacques Ouest

>Bureau 200

>Montréal (Québec)

>H2Y 1N9 Canada

>

>Telephone: 1 514 350 4300

>Facsimile: 1 514 350 4314

>Electronic mail: info@cec.org

>

The following references provide additional information on processing methodologies

and may be referenced in the process steps, below.

Beaubien, J., Cihlar, J., Simard, G., and Latifovic, R. (1999). Land cover from

multiple thematic mapper scenes using a new enhancement-classification methodology.

Journal of Geophysical Research, 104:27909 – 27920.

Cihlar, J., Xiao, Q., Beaubien, J., Fung, K., & Latifovic, R. (1998).

Classification by progressive generalization: a new automated methodology for

remote sensing multichannel data. International Journal of Remote Sensing

19:2685-2704.

Guindon et al. (2011). An improved method for the annual mapping of forest

disturbance across Canada based on MODIS 250m data and decision/regression trees.

32nd Canadian Symposium on Remote Sensing, June 13-16, Sherbrooke, Quebec.

Khlopenkov, K., & Trichtchenko, A. (2008). Implementation and evaluation of

concurrent gradient search method for reprojection of MODIS level 1B imagery. IEEE

Transaction on Geoscience and Remote Sensing, 46:2016–2027.

Latifovic, R., and Olthof, I. 2004. Accuracy assessment using sub-pixel fractional

error matrices of global land cover products derived from satellite data. Remote

Sensing of Environment, 90:153-165

Latifovic, R., Zhu, Z., Cihlar, J., Giri, C., & Olthof, I. (2004). Land cover

mapping of North and Central America - Global Land Cover 2000. Remote Sensing of

Environment, 89:116-127.

Luo, Y., Trishchenko, A., & Khlopenkov, K. (2008). Developing clear-sky, cloud and

cloud shadow mask for producing clear-sky composites at 250-meter spatial

resolution for the seven MODIS land bands over Canada and North America. Remote

Sensing of Environment, Volume 112:4167-4185.

Pavlic, G., and Latifovic, R. (2007). Canada-wide 250-m Water Fraction Coverage

derived from National Topographic Data Base. NRCan, ESS Program: Understanding

Canada from Space, Project: Land Surface Characterization.

Pouliot, D., Latifovic, R., Zabcic, N., Guindon, L., and  Olthof, I. (2013).

Development and assessment of a 250m spatial resolution MODIS annual land cover

time series (2000-2011) for Canada derived from change based updating. In review.

Timoney, K.P., La Roi, G.H., Zoltai, S.C., and Robinson, A.G. (1992)  The high

subarctic forest-tundra of northwestern Canada: Position, width, and vegetation

gradients in relation to climate. Arctic, 45, 1-9.

Trichtchenko, A., Luo Y., & Khlopenkov, K. (2006). A method for downscaling MODIS

land channels to 250m spatial resolution using adaptive regression and

normalization, Proceedings of SPIE – The International Society for Optical

Engineering v.6366 (2006) art. no. 636607. 8 pp.

Time_Period_of_Content:

Time_Period_Information:

Range_of_Dates/Times:

Beginning_Date: 200501

Ending_Date: 200512

Currentness_Reference: Ground condition

Status:

Progress: Complete

Maintenance_and_Update_Frequency: As needed

Spatial_Domain:

Bounding_Coordinates:

West_Bounding_Coordinate: 170

East_Bounding_Coordinate: -50

North_Bounding_Coordinate: 84

South_Bounding_Coordinate: 14

Keywords:

Theme:

Theme_Keyword_Thesaurus: GCMD Science keywords

Theme_Keyword: Land Surface > Land Use/Land Cover > Land Cover

Theme_Keyword: Land Surface > Surface Radiative Properties > Reflectance

Theme:

Theme_Keyword_Thesaurus: ISO 19115 Topic Category

Theme_Keyword: imageryBaseMapsEarthCover

Theme:

Theme_Keyword_Thesaurus: None

Theme_Keyword: Remote sensing

Theme_Keyword: Land cover

Theme_Keyword: MODIS

Theme_Keyword: Reflectance

Place:

Place_Keyword_Thesaurus: None

Place_Keyword: Mid-latitude

Place_Keyword: Western Hemisphere

Place_Keyword: Northern Hemisphere

Place_Keyword: North America

Place_Keyword: NAFTA

Place_Keyword: North America Free Trade Agreement

Place_Keyword: Canada

Place_Keyword: Mexico

Place_Keyword: United States

Access_Constraints: None.

Use_Constraints:

None.  Acknowledgement of Natural Resources Canada, Instituto Nacional de Estadística

y Geografía, and the U.S. Geological Survey is required in products derived from

these data.  Acknowledgement of the Commission for Environmental Cooperation would be

appreciated in products derived from these data.

Point_of_Contact:

Contact_Information:

Contact_Organization_Primary:

Contact_Organization: Canada Centre for Remote Sensing

Contact_Person: Rasim Latifovic

Contact_Address:

Address_Type: Mailing and physical address

Address: 588 Booth Street

City: Ottawa

State_or_Province: Ontario

Postal_Code: K1A 0Y7

Country: Canada

Contact_Voice_Telephone: 1 613 947 1816

Contact_Facsimile_Telephone: 1 613 947 1383

Contact_Electronic_Mail_Address: Rasim.Latifovic@nrcan.gc.ca

Point_of_Contact:

Contact_Information:

Contact_Organization_Primary:

Contact_Organization: Instituto Nacional de Estadística y Geografía (INEGI)

Contact_Position: Director of Natural Resources and Environment Information

Contact_Address:

Address_Type: Mailing and physical address

Address: Av. Héroe de Nacozari Sur 2301. Fraccionamiento Jardines del Parque

City: Aguascalientes

State_or_Province: Aguascalientes

Postal_Code: 20270

Country: Mexico

Contact_Voice_Telephone: 52 44 99 10 53 65

Contact_Electronic_Mail_Address: jose.ornelas@inegi.org.mx

Hours_of_Service: 0800 - 1600 (-6h GMT)

Point_of_Contact:

Contact_Information:

Contact_Organization_Primary:

Contact_Organization: U.S. Geological Survey

Contact_Person: Collin Homer

Contact_Address:

Address_Type: Mailing and physical address

Address: USGS, Earth Resources Observation and  Science (EROS) Center

Address: Mundt Federal Building

Address: 47914 252nd Street

City: Sioux Falls

State_or_Province: South Dakota

Postal_Code: 57198-0001

Country: USA

Contact_Voice_Telephone: 1 605 594 2714

Contact_Electronic_Mail_Address: homer@usgs.gov

Data_Quality_Information:

Attribute_Accuracy:

Attribute_Accuracy_Report:

Canada:

An accuracy assessment was performed on the Canadian data at the 12-class IGBP

level.

To assist in tree life-form identification where crown shape was difficult to

interpret, reference data were obtained through visual assessment of high-

resolution image data in Google Earth, supported by Landsat data.  Within the high-

resolution imagery, random sampling was used to generate 838 reference samples.

Each sample was interpreted and assigned a primary label, a secondary label and a

confidence level.  The primary label referred to the most likely class assigned to

a pixel according to the interpreter, while the secondary label referred to a

second class that could also be considered acceptable in cases where ambiguity

existed in the interpretation or where more than one label could be deemed correct.

Examples of the latter occurred mainly along transition zones where two classes

occurred almost equally within a 250m pixel.

The confidence level was assigned based on the ease with which the land cover was

visually interpreted and on the number of classes present in the 250m pixel.  A

high confidence was assigned to pixels where the high-resolution Google Earth image

was acquired at an appropriate time of year for the class in question (for example,

summer to fall for deciduous), and where one class clearly dominated the land cover

within the pixel.

For the primary label only, overall accuracy was nearly 59% with a Kappa of 0.54.

Overall accuracy increased to 69% and the Kappa increased to 0.65 when considering

either the primary or secondary label as being correct.  For primary labels

interpreted with high confidence, representing 365 of 838 reference samples,

overall accuracy increased again to 75% with a Kappa of 0.72.  The increase in

classification accuracy with high reference-data interpretation confidence was a

function of greater land cover homogeneity within the reference data footprint and

less ambiguity largely due to reference image quality.

The analysis of the classification error matrix revealed known sources of spectral

confusion among land cover classes.  Adjacent forest classes tended to be confused,

as well as deciduous forest, shrubland, shrub-covered wetlands, and certain

croplands.  These classes were difficult to separate with spectral data alone, due

to all classes being primary broadleaved deciduous and the known radiometric

saturation at leaf area index levels in the range of 3-5, which was achieved in all

three classes.  Other issues arose with the herbaceous class, which was either

conifer (according to reference data along the treeline) consisting of open treed

areas with herbaceous understory, or low biomass croplands.  Confusion between

herb, shrub, and deciduous was also due to relatively small disturbance patch sizes

of cuts, primarily in British Columbia, and the point-spread function of the

sensor, leading to reduced detection of objects smaller than seven pixels.

Finally, the lichen/moss class was either herbaceous or wetland according to the

reference data, due to the prevalence of both lichen and moss in  certain wetlands

and the low biomass of  both the lichen/moss and herbaceous classes.

A theoretical analysis of accuracy assessment of global land cover products over

Canada in Latifovic and Olthof (2004) examined accuracy as a function of land cover

homogeneity and thematic resolution.  Based on land cover homogeneity in Canada,

determined from a representative sample of medium resolution (~30m) land cover

products, it was determined that a theoretical accuracy of 70-75% at a 16-land-

cover-class thematic level should be achievable depending on the degree of labeling

error.  Accuracy for the 2005 Canadian land cover data is slightly lower when

considering only the primary reference label and is in the 70% range when

considering the primary or secondary label.  This lower-than-theoretically-

achievable accuracy is due to errors in the reference data labels as is evident

from the increase in accuracy when both labels are considered.  When labeling error

is reduced further by considering only reference data that were interpreted with

100% confidence, the accuracy climbs to 75%.  While this is on the upper range of

the theoretically achievable accuracy, indicating minimal to no labeling error, it

should be noted that the current assessment was done at a 12-class thematic level,

which should increase the achievable accuracy and suggests that some labeling error

is still present in the data set.

Mexico:

No formal accuracy assessment has been performed on the Mexican data,

however, land cover proportions and distributions agree well with independent land

cover and land use estimations of the “Mapa de Vegetación y Uso de Suelo”, Serie

III of 2007 by INEGI.

USA:

No formal accuracy assessment has been performed for data products in the United

States and this land cover data set should be considered provisional until a formal

accuracy assessment is conducted.   During processing, the regression tree

algorithm employed for land cover classification offers a cross-validation option

for assessing classification and prediction reliability.  Cross-validation provides

relatively reliable estimates for land cover model performance if the reference

data used for cross-validation are collected based on a statistically valid

sampling design.  The cross-validation values for each zone were used to guide

refinements to model training data and to improve classification results.  The

quantitative value reported describes an average cross-validation value across

several mapped zones and is not implied as a formal indicator of product accuracy.

Quantitative_Attribute_Accuracy_Assessment:

Attribute_Accuracy_Value: 70

Attribute_Accuracy_Explanation:

The above listed value is the overall accuracy obtained for the Canada land cover

data using the reference data set.

Quantitative_Attribute_Accuracy_Assessment:

Attribute_Accuracy_Value: 69

Attribute_Accuracy_Explanation:

For the United States: Informal accuracy indicator based on average values of

cross-validation reports from a regression tree algorithm.

Quantitative_Attribute_Accuracy_Assessment:

Attribute_Accuracy_Value: 85

Attribute_Accuracy_Explanation:

The above listed value is the overall accuracy obtained for the Mexican land

cover data using a subset of 20% of the training samples from the decision tree

model.

Logical_Consistency_Report:

No tests for logical consistency have been performed on this data set.

Completeness_Report:

Data completeness reflects the content of the original MODIS data.

Lineage:

Source_Information:

Source_Citation:

Citation_Information:

Originator:

Canada Centre for Remote Sensing, Earth Sciences Sector, Natural Resources

Canada

Publication_Date: 2008

Title: MODIS Level 1B 250m swath data, 5-min granules

Geospatial_Data_Presentation_Form: Raster digital data

Publication_Information:

Publication_Place: Ottawa, Ontario, Canada

Publisher: NASA

Online_Linkage: http://ladsweb.nascom.nasa.gov/

Type_of_Source_Media: Online

Source_Time_Period_of_Content:

Time_Period_Information:

Single_Date/Time:

Calendar_Date: 2005

Source_Currentness_Reference: Ground condition

Source_Citation_Abbreviation: MOD02QKM

Source_Contribution: Spatial and attribute information

Source_Information:

Source_Citation:

Citation_Information:

Originator:

Canada Centre for Remote Sensing, Earth Sciences Sector, Natural Resources

Canada

Publication_Date: 2008

Title: MODIS Level 1B 500m swath data, 5-min granules

Geospatial_Data_Presentation_Form: Raster digital data

Publication_Information:

Publication_Place: Ottawa, Ontario, Canada

Publisher: NASA

Online_Linkage: http://ladsweb.nascom.nasa.gov/

Type_of_Source_Media: Online

Source_Time_Period_of_Content:

Time_Period_Information:

Single_Date/Time:

Calendar_Date: 2005

Source_Currentness_Reference: Ground condition

Source_Citation_Abbreviation: MOD02HKM

Source_Contribution: Spatial and attribute information

Source_Information:

Source_Citation:

Citation_Information:

Originator:

Canada Centre for Remote Sensing, Earth Sciences Sector, Natural Resources

Canada

Publication_Date: 2008

Title: MODIS Level 1 Geolocation information, 1km swath data, 5-min granules

Geospatial_Data_Presentation_Form: Raster digital data

Publication_Information:

Publication_Place: Ottawa, Ontario, Canada

Publisher: NASA

Online_Linkage: http://ladsweb.nascom.nasa.gov/

Type_of_Source_Media: Online

Source_Time_Period_of_Content:

Time_Period_Information:

Single_Date/Time:

Calendar_Date: 2005

Source_Currentness_Reference: Ground condition

Source_Citation_Abbreviation: MOD03

Source_Contribution: Spatial and attribute information

Source_Information:

Source_Citation:

Citation_Information:

Originator:

Canada Centre for Remote Sensing, Earth Sciences Sector, Natural Resources

Canada

Publication_Date: 2009

Title: MODIS Monthly composites

Geospatial_Data_Presentation_Form: Raster digital data

Type_of_Source_Media: Internal files

Source_Time_Period_of_Content:

Time_Period_Information:

Single_Date/Time:

Calendar_Date: 2005

Source_Currentness_Reference: Date of data compilation

Source_Citation_Abbreviation: NA Monthly Composites

Source_Contribution: Spatial and attribute information

Source_Information:

Source_Citation:

Citation_Information:

Originator: Various

Publication_Date: Unknown

Title: Ancillary data

Other_Citation_Details:

The following data sets were used to aid cluster labelling:

>Data set                           Source

>--------------------------------------------------------------------------

>Water Fraction Map of Canada       http://www.geobase.ca

>  assembled from the National

>  Topographic Data Base (NTDB)

>

>Canada Digital Elevation Data,     http://www.geobase.ca

>  Level 1

>

>National Road Network, Canada      http://www.geobase.ca

>

>Satellite database for the land    CCRS

>  cover of Canada is a sample

>  of LANDSAT TM/ETM+ scenes

>  representing the distribution

>  of land cover across Canada.

>

>Earth Observation for Sustainable  CFS

>  Development (EOSD) of  Forests

>  Land Cover Classification

>

>National Land and Water            http://www.agr.gc.ca/eng/?id=1343071073307#a9

>  Information System (NLWIS)

>  Land Cover for agricultural

>  regions of Canada, circa 2000

>

>Circa-2000 Northern Land Cover     http://geogratis.cgdi.gc.ca

>  of Canada

>

>Classification of Agricultural     Agricultural Financial Services

>  Lands in Alberta, Saskatchewan     Corporation Saskatchewan Crop

>  and the Peace Region of            Insurance Corporation

>  British Columbia                   British Columbia Ministry of

>                                     Agriculture and Lands (Business

>                                     Risk management Division)

>

>Northern Treeline (Timoney         http://data.arcticatlas.org

>  et al., 1992)

>

>North American Regional            http://nomads.ncdc.noaa.gov

>  Reanalysis, Daily Dataset

>  Degree days

>

>Fire data base                     CCRS

>

>Ground truth data                  CCRS

Publication_Information:

Publication_Place: Various

Publisher: Various

Type_of_Source_Media: Various

Source_Time_Period_of_Content:

Time_Period_Information:

Single_Date/Time:

Calendar_Date: 2009

Source_Currentness_Reference: Date of acquisition

Source_Citation_Abbreviation: Canada ancillary data

Source_Contribution: Attribute information

Source_Information:

Source_Citation:

Citation_Information:

Originator: Various

Publication_Date: Unknown

Title: Ancillary data

Other_Citation_Details:

The following data sets were used to aid cluster labelling:

>Data set                           Source

>--------------------------------------------------------------------------

>Total days of precipitation per    CONAFOR

>  year (1970 – 2000)

>

>Total of evaporation per year in   CONAFOR

>  mm (1970 – 2000)

>

>Maximum temperature average per    CONAFOR

>  year in °C (1970 – 2000)

>

>Monthly average temperature per    CONAFOR

>  year in °C (1970 – 2000)

>

>Minimum monthly average            CONAFOR

>  temperature per year in °C

>  (1970 – 2000)

>

>Digital Elevation Model of         CONAFOR

>  Mexico 250m

>

>Classification of Agricultural     Colegio de Posgraduados

>  Lands Mexico                       (COLPOS)

>                                   Secretaría de Agricultura,

>                                     Ganadería, Desarrollo Rural,

>                                     Pesca y Alimentación de

>                                     México (SAGARPA)

>

>Ground truth data for forested     CONAFOR

>  areas in Mexico

>

>Urban areas in Mexico              INEGI

>

>Mapa de Uso de Suelo y Vegetacion  INEGI

>

Publication_Information:

Publication_Place: Various

Publisher: Various

Type_of_Source_Media: Various

Source_Time_Period_of_Content:

Time_Period_Information:

Single_Date/Time:

Calendar_Date: 2009

Source_Currentness_Reference: Date of acquisition

Source_Citation_Abbreviation: Mexico ancillary data

Source_Contribution: Attribute information

Source_Information:

Source_Citation:

Citation_Information:

Originator: NASA

Publication_Date: 2008

Title: Landsat data

Geospatial_Data_Presentation_Form: Raster digital data

Publication_Information:

Publication_Place: Washington, DC

Publisher: NASA

Type_of_Source_Media: Online

Source_Time_Period_of_Content:

Time_Period_Information:

Single_Date/Time:

Calendar_Date: 2005

Source_Currentness_Reference: Ground condition

Source_Citation_Abbreviation: Landsat

Source_Contribution: Attribute information

Source_Information:

Source_Citation:

Citation_Information:

Originator: U.S. Geological Survey

Publication_Date: 2008

Title: DEM (250m) composed of 1 band of Continuous Variable Type

Geospatial_Data_Presentation_Form: Raster digital data

Publication_Information:

Publication_Place: Reston, VA

Publisher: U.S. Geological Survey

Type_of_Source_Media: Online

Source_Time_Period_of_Content:

Time_Period_Information:

Single_Date/Time:

Calendar_Date: 2005

Source_Currentness_Reference: Ground condition

Source_Citation_Abbreviation: DEM

Source_Contribution: Attribute information

Source_Information:

Source_Citation:

Citation_Information:

Originator: Canada Centre for Remote Sensing

Publication_Date: Unknown

Title: 2005 Land Cover Map of North America at 250m: Canada

Geospatial_Data_Presentation_Form: Raster digital data

Publication_Information:

Publication_Place: Ottawa, Ontario, Canada

Publisher: Canada Centre for Remote Sensing

Type_of_Source_Media: Internal file

Source_Time_Period_of_Content:

Time_Period_Information:

Single_Date/Time:

Calendar_Date: 2005

Source_Currentness_Reference: Ground condition

Source_Citation_Abbreviation: CCRS_NLCD

Source_Contribution: Spatial and attribute information

Source_Information:

Source_Citation:

Citation_Information:

Originator:

Comisión Nacional para el Conocimiento y Uso de la Biodiversidad (CONABIO)

Originator: Comisión Nacional Forestal (CONAFOR)

Originator: Insituto Nacional de Estadística y Geografía (INEGI)

Publication_Date: Unknown

Title: Land Cover Map of Mexico for 2005 at 250m

Geospatial_Data_Presentation_Form: Raster digital data

Publication_Information:

Publication_Place: Mexico City, Mexico

Publisher: Insituto Nacional de Estadística y Geografía

Type_of_Source_Media: Internal file

Source_Time_Period_of_Content:

Time_Period_Information:

Single_Date/Time:

Calendar_Date: 2005

Source_Currentness_Reference: Ground condition

Source_Citation_Abbreviation: INEGI_NLCD

Source_Contribution: Spatial and attribute information

Source_Information:

Source_Citation:

Citation_Information:

Originator: U.S. Geological Survey

Publication_Date: Unknown

Title: 2005 North American Land Cover Database for the United States

Geospatial_Data_Presentation_Form: Raster digital data

Publication_Information:

Publication_Place: Sioux Falls, South Dakota, USA

Publisher: U.S. Geological Survey

Type_of_Source_Media: Internal file

Source_Time_Period_of_Content:

Time_Period_Information:

Single_Date/Time:

Calendar_Date: 2005

Source_Currentness_Reference: Ground condition

Source_Citation_Abbreviation: USGS_NLCD

Source_Contribution: Spatial and attribute information

Source_Information:

Source_Citation:

Citation_Information:

Originator:

Canada Centre for Remote Sensing (CCRS), Earth Sciences Sector, Natural

Resources Canada

Originator:

Comisión Nacional para el Conocimiento y Uso de la Biodiversidad (CONABIO)

Originator: Comisión Nacional Forestal (CONAFOR)

Originator: Insituto Nacional de Estadística y Geografía (INEGI)

Originator: U.S. Geological Survey (USGS)

Publication_Date: 2010

Title: 2005 Land Cover of North America at 250 meters

Edition: 1.0

Geospatial_Data_Presentation_Form: Raster digital data

Publication_Information:

Publication_Place: Ottawa, Ontario, Canada

Publisher: Canada Centre for Remote Sensing (CCRS)

Publication_Information:

Publication_Place: Sioux Falls, South Dakota, USA

Publisher: U.S. Geological Survey

Publication_Information:

Publication_Place: Mexico City, Mexico

Publisher: INEGI

Publication_Information:

Publication_Place: Montréal, Québec, Canada

Publisher: Commission for Environmental Cooperation

Online_Linkage: http://www.cec.org/naatlas/

Type_of_Source_Media: Online

Source_Time_Period_of_Content:

Time_Period_Information:

Single_Date/Time:

Calendar_Date: 2005

Source_Currentness_Reference: Ground condition

Source_Citation_Abbreviation: LandCover2005_v1

Source_Contribution: Spatial and attribute information

Source_Information:

Source_Citation:

Citation_Information:

Originator: Google

Publication_Date: Unknown

Title: Google Earth

Online_Linkage: <http://earth.google.com/>

Type_of_Source_Media: Online

Source_Time_Period_of_Content:

Time_Period_Information:

Single_Date/Time:

Calendar_Date: Unknown

Source_Currentness_Reference: Publication date

Source_Citation_Abbreviation: Google Earth

Source_Contribution: Spatial and attribute information.

Source_Information:

Source_Citation:

Citation_Information:

Originator: U.S. Geological Survey

Publication_Date: 2008

Title: 2001 North American Land Cover Database for the United States

Online_Linkage: <http://www.mrlc.gov/nlcd01_data.php>

Geospatial_Data_Presentation_Form: Raster digital data

Publication_Information:

Publication_Place: Sioux Falls, South Dakota, USA

Publisher: U.S. Geological Survey

Type_of_Source_Media: Online

Source_Time_Period_of_Content:

Time_Period_Information:

Single_Date/Time:

Calendar_Date: 2001

Source_Currentness_Reference: Ground condition

Source_Citation_Abbreviation: USGS_NLCD_2001

Source_Contribution: Spatial and attribute information.

Source_Information:

Source_Citation:

Citation_Information:

Originator:

U.S. Geological Survey, Center for Earth Resources Observation and Science

(EROS)

Originator:

U.S. Department of Agriculture, Forest Service, Remote Sensing Applications

Center (RSAC)

Publication_Date: 20121003

Title: Multi-Temporal Burn Severity Fire Perimeters

Online_Linkage: <http://www.mtbs.gov/dataquery/individualfiredata.html>

Type_of_Source_Media: Online

Source_Time_Period_of_Content:

Time_Period_Information:

Single_Date/Time:

Calendar_Date: 20121003

Source_Currentness_Reference: Publication date

Source_Citation_Abbreviation: MTBS

Source_Contribution: Spatial and attribute information.

Process_Step:

Process_Description:

A series of monthly MODIS composites were produced prior to the compilation of

land cover data for each country.

Input data used in land cover mapping were generated from the following original

Level 1B MODIS data (collection 5): a) MOD02QKM - Level 1B 250m swath data, 5-min

Granules; b) MOD02HKM - Level 1B 500m swath data, 5-min granules; c) MOD03 –

Level 1 Geolocation information, 1km swath data, 5-min granules.  All input data

are available from the NASA Level 1 and Atmosphere Archive and Distribution

System (LAADS), http://ladsweb.nascom.nasa.gov/

The MODIS 500m land channels (B3-B7) were downscaled to 250m spatial resolution

as described by Trishchenko et al. (2006) and then re-projected into Lambert

Azimuthal Equal Area projection using software developed at the CCRS (Khlopenkov

et al., 2008). Two original 250m MODIS channels and five downscaled 250m MODIS

land channels were assembled into North-America-wide 1  10-day composites following

the methods developed in Luo et al., (2008).

MODIS 10-day composites were further processed to reduce data noise due to

different viewing conditions and remaining cloud and atmosphere contamination.  A

temporal rank filter was applied, calculated as the maximum of minimum values for

two overlapping moving windows.  For this implementation, a window size of three

ten-day composites was used with one pixel overlap between windows.  After rank

filtering, a procedure to detect residual spikes in the data was applied based on

a neighbourhood comparison.  Within a moving window of 3 composites, if the

neighbouring pixels were both above or below the centre pixel by 25%, the centre

pixel was linearly interpolated from its neighbours.  The filtered 10-day

composites were averaged to generate monthly composites.  The processed data set

contained 12 monthly composites of channels 1 (B1, VIS) and 2 (B2, near-infrared

NIR) at 250m spatial resolution and five channels designed for land applications

(bands B3 to B7) resampled to 250m spatial resolution.

Source_Used_Citation_Abbreviation: MOD02QKM

Source_Used_Citation_Abbreviation: MOD02HKM

Source_Used_Citation_Abbreviation: MOD03

Process_Date: 2009

Source_Produced_Citation_Abbreviation: NA Monthly composites

Process_Step:

Process_Description:

Canadian land cover was generated following the procedure described in Latifovic

et al. (2004) which combines the Enhanced Classification Method (ECM, Beaubien et

al.,  1999) and Classification by Progressive Generalization (CPG, Cihlar et al.,

1998 and  Latifovic et al., 1999).  The principal characteristic of this

classification approach is the preservation of image information and

incorporation of human contextual reasoning for land cover labeling.  It includes

the following generalization steps: 1) initial clustering, 2) cluster

agglomeration based on spectral similarity and cluster size , 3) cluster

agglomeration based on thematic similarity, and 4) cluster labeling.  Initial

clusters were generated using the K-mean classifier with bands 1, 2, and 6 from a

2005 summer composite created by averaging composites for July and August.

Several ancillary GIS layers and other land cover products over regions of Canada

were consulted to refine and improve land cover labeling.

Source_Used_Citation_Abbreviation: NA Monthly Composites

Source_Used_Citation_Abbreviation: Canada ancillary data

Process_Date: 2009

Source_Produced_Citation_Abbreviation: CCRS_NLCD

Process_Step:

Process_Description:

For Mexico, the monthly composites of MODIS-Terra top-of-the-atmosphere (TOA)

surface radiance data for the first seven spectral bands and the Normalized

Difference Vegetation Index (NDVI) were employed for the land cover

classification process.  For each spectral band and the NDVI, the mean, standard

deviation, minimum, maximum, and range between minimum and maximum were computed

over the year, half-year, four-month, and quarter-year periods.  This resulted in

400 metrics derived from MODIS data.  In addition, ancillary data were employed

including a digital elevation model and derivatives such as slope and aspect,

mean, maximum and minimum temperature; total of precipitation and days with

precipitation; and the number of green days.

Sample data were derived from various sources, including data of the Mexican

Forest Inventory, detailed cropland data, urban cover, and additional digitized

points from reference sources;  20% of the samples were exclusively used for

validation.

The land cover type identification was achieved using a Decision Tree Classifier

(DTC).  C5.0 classification trees were generated for the training data.  Because

exclusively continuous variables were employed, the tree only contains binary

splits.  Boosting with ten iterations was employed.  The trees were applied to

the data and the confidences were computed which were used as weights for

combining the individual classifications.  Several classifications were generated

based on combinations of 1) input data - monthly TOA reflectances, generated

metrics and ancillary data; and 2) complete or subset data sets of training data.

Visual and statistical assessments were carried out to obtain the final

classification.  A distance layer was generated based on an ecoregions map to

combine results of the different classifications; only selected classifications

were selected for regions where they performed better visually and statically.

Finally, a water mask was superimposed; the mask was derived from the “Mapa de

Uso de Suelo y Vegetacion” from INEGI.

Source_Used_Citation_Abbreviation: NA Monthly Composites

Source_Used_Citation_Abbreviation: Mexico ancillary data

Process_Date: 2009

Source_Produced_Citation_Abbreviation: INEGI_NLCD

Process_Step:

Process_Description:

United States classification was achieved by use of a classification and

decision tree method (DT) using a combination of Landsat imagery and ancillary

data (DEM).  The specific DT program employed is called C5, which implements a

gain ratio criterion in tree development and pruning (Quinlan, 1993).  C5 also

implemented several advanced features that can aid and improve land cover

classification, including boosting and cross-validation.  Boosting is a technique

for improving classification accuracy, while cross-validation can provide a

certain level of estimation regarding the land cover classification quality.  In

addition, C5 can generate a confidence estimate for each classified pixel and

record the associated classification logic in a text file that can be readily

interpreted and incorporated into a metadata system.  To conduct the land cover

classification using DT, a large quantity of training data was required.  For

North America mapping , training data were collected from several combined

sources including NLCD 2001 land cover, GAP land cover, NOAA Coastal Change and

Analysis land cover, LANDFIRE  land cover and other data sources.

Following the development of the best classification through decision tree

modeling, additional steps (described in an unpublished technical document; see

the USGS point of contact if additional information is needed) were required to

complete the final land cover product.  Finally, visual inspection of the

classification was made with areas/pixels that were wrongly classified delineated

first as an "area of interest" (AOI); subsequently, limited manual editing was

done to eliminate the classification error within the AOI.

Source_Used_Citation_Abbreviation: NA Monthly composites

Source_Used_Citation_Abbreviation: Landsat

Source_Used_Citation_Abbreviation: DEM

Process_Date: 2009

Source_Produced_Citation_Abbreviation: USGS_NLCD

Process_Step:

Process_Description:

To produce the seamless land cover map of North America, the three national

land cover products (Canada, Mexico, USA) were edge matched along the borders

using an object-oriented segmentation technique.  Land cover regions that crossed

borders were reclassified using the majority class segment.

Source_Used_Citation_Abbreviation: CCRS_NLCD

Source_Used_Citation_Abbreviation: USGS_NLCD

Source_Used_Citation_Abbreviation: INEGI_NLCD

Process_Date: 2009

Source_Produced_Citation_Abbreviation: LandCover2005_v1

Process_Step:

Process_Description:

In Version 1 of the 2005 Land Cover data there were three classes that were

incorrectly classified in Mexico: urban, water, and cropland.

Urban area misclassifications included both areas where the extent was

overestimated (e.g. the state of Puebla) and areas where the extent was

underestimated (mainly in northern Mexico.)  Areas that were mis-estimated were

assigned to the most appropriate land cover class by visual interpretation;

Google Earth was used for verification and visual analysis.

Most water misclassifications were underestimations.  It was important to correct

this problem to allow for accurate change detection between this and later data

sets.   The waterbodies were identified on MODIS imagery and both size and shape

of the areas was corrected.  In addition, waterbodies that were not previously

mapped were digitized where appropriate.  A few waterbodies were reduced in size

or eliminated but this was rare.  For waterbodies that experience fluctuating

water levels due to seasonal rains or to water storage and release in artificial

lakes, the highest water mark, as indicated by the post-rainy season image

(October),  was used to map the waterbody size and shape.

There were two problems with the classification of cropland.  First, the

definition of the cropland class is very broad and includes extensive rangeland

and extensive agriculture.   Second, in the Gulf Coast region cropland is

spatially overestimated, but because the actual spatial patterns cannot be mapped

from MODIS images, correction is difficult.  As a result, cropland

classifications were not modified in the version 2.

Source_Used_Citation_Abbreviation: LandCover2005_v1

Source_Used_Citation_Abbreviation: NA Monthly Composites

Source_Used_Citation_Abbreviation: Google Earth

Process_Date: 2013

Process_Step:

Process_Description:

In Version 1 of the 2005 Land Cover data, Alaska was not correctly aligned with

the MODIS source data.  A new version of the land cover for Alaska was produced

to fix the alignment problem.  The land cover was remodeled using properly

aligned imagery and new training data for the model.  Some small areas in the

extreme north and south of Alaska were missing data and these areas were added

from cross-walked 2001 NLCD data and the MODIS 2010 data (it was assumed that

there were land cover changes in these areas between 2005 and 2010.)

The land cover information for the conterminous United States was adjusted for

more accurate classification.  In particular, an effort was made to improve the

mapping of areas affected by fire, using the MTBS data.  Burned areas were

reclassified as necessary, based on the length of time since the fire occurred.

Shrub areas in the southwestern U.S. which had the wrong Level II class (Class 8

instead of Class 7) were adjusted.

Source_Used_Citation_Abbreviation: LandCover2005_v1

Source_Used_Citation_Abbreviation: NA Monthly composites

Source_Used_Citation_Abbreviation: DEM

Source_Used_Citation_Abbreviation: USGS_NLCD_2001

Source_Used_Citation_Abbreviation: MTBS

Process_Date: 2013

Process_Step:

Process_Description:

Since the development of the Version 1 data for Canada, further quality

assessment and input from users suggested modifications in some areas.  The

original source data were re-analyzed; major changes included reduction of treed

wetland along the western and eastern coasts, correction of water/shadow

confusion in regions of complex terrain, correction of snow/water confusion in

the far north, improved disturbance representation based on change detection

(Guindon et al., 2011), improved forest mapping in the prairies region, some

minor changes to urban areas, and some local improvements to wetland mapping.

Source_Used_Citation_Abbreviation: NA Monthly Composites

Source_Used_Citation_Abbreviation: Canada ancillary data

Source_Used_Citation_Abbreviation: LandCover2005_v1

Process_Date: 2013

Process_Step:

Process_Description:

The updated data sets from Canada, Mexico, and the U.S. were combined into one

data set covering all of North America.

Process_Date: 2013

Spatial_Data_Organization_Information:

Direct_Spatial_Reference_Method: Raster

Raster_Object_Information:

Raster_Object_Type: Pixel

Row_Count: 35000

Column_Count: 37000

Spatial_Reference_Information:

Horizontal_Coordinate_System_Definition:

Planar:

Map_Projection:

Map_Projection_Name: Lambert Azimuthal Equal Area

Lambert_Azimuthal_Equal_Area:

Longitude_of_Projection_Center: -100.00

Latitude_of_Projection_Center: 45.00

False_Easting: 0.0

False_Northing: 0.0

Planar_Coordinate_Information:

Planar_Coordinate_Encoding_Method: Row and column

Coordinate_Representation:

Abscissa_Resolution: 250.0

Ordinate_Resolution: 250.0

Planar_Distance_Units: Meters

Geodetic_Model:

Horizontal_Datum_Name: None

Ellipsoid_Name: Sphere

Semi-major_Axis: 6370977

Denominator_of_Flattening_Ratio: 1.0

Entity_and_Attribute_Information:

Detailed_Description:

Entity_Type:

Entity_Type_Label: Land cover classification grid cell

Entity_Type_Definition:

Any of the data elements in the 2005 North American land cover file

Entity_Type_Definition_Source: North American Atlas GIS Group

Attribute:

Attribute_Label: Land cover classification grid cell value

Attribute_Definition: The value is an indication of the land cover class

Attribute_Definition_Source: North American Atlas GIS Group

Attribute_Domain_Values:

Enumerated_Domain:

Enumerated_Domain_Value: 1

Enumerated_Domain_Value_Definition:

Temperate or sub-polar needleleaf forest.  Forests generally taller than

three meters and more than 20 percent of total vegetation cover.  This type

occurs across the northern United States, Canada and mountainous zones of

Mexico.  The tree crown cover contains at least 75 percent of needleleaved

species.  Land Cover Classification System (LCCS) code: 20134.

Enumerated_Domain_Value_Definition_Source:

North American Atlas GIS Group

Attribute_Domain_Values:

Enumerated_Domain:

Enumerated_Domain_Value: 2

Enumerated_Domain_Value_Definition:

Sub-polar taiga needleleaf forest.  Forest and woodlands with trees generally

taller than three meters and more than 5 percent of total vegetation cover

with shrubs and lichens commonly present in the understory.  The tree crown

cover contains at least 75 percent of needleleaved species.  This type occurs

across Alaska and northern Canada and may consist of treed muskeg or

wetlands.  Forest canopies are variable and often sparse, with generally

greater tree cover in the southern latitude parts of the zone than the north.

LCCS code: 20229.

Enumerated_Domain_Value_Definition_Source:

North American Atlas GIS Group

Attribute_Domain_Values:

Enumerated_Domain:

Enumerated_Domain_Value: 3

Enumerated_Domain_Value_Definition:

Tropical or sub-tropical broadleaf evergreen forest.  Forests generally

taller than five meters and more than 20 percent of total vegetation cover.

These occur in the southern United States and Mexico.  These forests have

greater than 75 percent of tree crown cover represented by evergreen species.

LCCS code: 20090.

Enumerated_Domain_Value_Definition_Source:

North American Atlas GIS Group

Attribute_Domain_Values:

Enumerated_Domain:

Enumerated_Domain_Value: 4

Enumerated_Domain_Value_Definition:

Tropical or sub-tropical broadleaf deciduous forest.  Forests generally

taller than five meters and more than 20 percent of total vegetation cover.

These occur in the southern United States and Mexico.  These forests have

greater than 75 percent of tree crown cover represented by deciduous species.

LCCS code: 20132.

Enumerated_Domain_Value_Definition_Source:

North American Atlas GIS Group

Attribute_Domain_Values:

Enumerated_Domain:

Enumerated_Domain_Value: 5

Enumerated_Domain_Value_Definition:

Temperate or sub-polar broadleaf deciduous forest.  Forests generally taller

than three meters and more than 20 percent of total vegetation cover.  These

occur in the northern United States, Canada and mountainous zones of Mexico.

These forests have greater than 75 percent of tree crown cover represented by

deciduous species.  LCCS code: 20227.

Enumerated_Domain_Value_Definition_Source:

North American Atlas GIS Group

Attribute_Domain_Values:

Enumerated_Domain:

Enumerated_Domain_Value: 6

Enumerated_Domain_Value_Definition:

Mixed forest.  Forests generally taller than three meters and more than 20

percent of total vegetation cover.  Neither needleleaf nor broadleaf tree

species occupy more than 75 percent of total tree cover, but are co-dominant.

LCCS codes: 20092, 20090, 20134, 20132, 20229, 20227.

Enumerated_Domain_Value_Definition_Source:

North American Atlas GIS Group

Attribute_Domain_Values:

Enumerated_Domain:

Enumerated_Domain_Value: 7

Enumerated_Domain_Value_Definition:

Tropical or sub-tropical shrubland.  Areas dominated by woody perennial

plants with persistent woody stems less than five meters tall and typically

greater than 20 percent of total vegetation.  This class occurs across the

southern United States and Mexico.  LCCS code: 21450-13476.

Enumerated_Domain_Value_Definition_Source:

North American Atlas GIS Group

Attribute_Domain_Values:

Enumerated_Domain:

Enumerated_Domain_Value: 8

Enumerated_Domain_Value_Definition:

Temperate or sub-polar shrubland.  Areas dominated by woody perennial plants

with persistent woody stems less than three meters tall and typically greater

than 20 percent of total vegetation.  This class occurs across the northern

United States, Canada and highlands of Mexico.  LCCS code: 21450-12050.

Enumerated_Domain_Value_Definition_Source:

North American Atlas GIS Group

Attribute_Domain_Values:

Enumerated_Domain:

Enumerated_Domain_Value: 9

Enumerated_Domain_Value_Definition:

Tropical or sub-tropical grassland.  Areas dominated by graminoid or

herbaceous vegetation generally accounting for greater than 80 percent of

total vegetation cover.  These areas are not subject to intensive management

such as tilling, but can be utilized for grazing.  This class occurs across

the southern United States and Mexico.  LCCS code: 21669.

Enumerated_Domain_Value_Definition_Source:

North American Atlas GIS Group

Attribute_Domain_Values:

Enumerated_Domain:

Enumerated_Domain_Value: 10

Enumerated_Domain_Value_Definition:

Temperate or sub-polar grassland.  Areas dominated by graminoid or herbaceous

vegetation, generally accounting for greater than 80 percent of total

vegetation cover.  These areas are not subject to intensive management such

as tilling, but can be utilized for grazing.  This class occurs across

Canada, United States and highlands of Mexico.  LCCS code: 21537-12212.

Enumerated_Domain_Value_Definition_Source:

North American Atlas GIS Group

Attribute_Domain_Values:

Enumerated_Domain:

Enumerated_Domain_Value: 11

Enumerated_Domain_Value_Definition:

Sub-polar or polar shrubland-lichen-moss.  Areas dominated by dwarf shrubs

with lichen and moss typically accounting for at least 20 percent of total

vegetation cover.  This class occurs across northern Canada and Alaska.  LCCS

codes: 20022-12050, 21454-12212, 21439-3012.

Enumerated_Domain_Value_Definition_Source:

North American Atlas GIS Group

Attribute_Domain_Values:

Enumerated_Domain:

Enumerated_Domain_Value: 12

Enumerated_Domain_Value_Definition:

Sub-polar or polar grassland-lichen-moss.  Areas dominated by grassland with

lichen and moss typically accounting for at least 20 percent of total

vegetation cover.  This class occurs across northern Canada and Alaska.  LCCS

codes: 21454-12212, 20022-12050, 21439-3012.

Enumerated_Domain_Value_Definition_Source:

North American Atlas GIS Group

Attribute_Domain_Values:

Enumerated_Domain:

Enumerated_Domain_Value: 13

Enumerated_Domain_Value_Definition:

Sub-polar or polar barren-lichen-moss.  Areas dominated by a mixture of bare

areas with lichen and moss that typically account for at least 20 percent of

total vegetation cover.  This class occurs across northern Canada and Alaska.

LCCS codes: 21468, 21454-12212, 20022-12050.

Enumerated_Domain_Value_Definition_Source:

North American Atlas GIS Group

Attribute_Domain_Values:

Enumerated_Domain:

Enumerated_Domain_Value: 14

Enumerated_Domain_Value_Definition:

Wetland.  Areas dominated by perennial herbaceous and woody wetland

vegetation which is influenced by the water table at or near surface over

extensive periods of time.  This includes marshes, swamps, bogs, mangroves,

etc., either coastal or inland where water is present for a substantial

period annually.  LCCS codes: 42349, 41809.

Enumerated_Domain_Value_Definition_Source:

North American Atlas GIS Group

Attribute_Domain_Values:

Enumerated_Domain:

Enumerated_Domain_Value: 15

Enumerated_Domain_Value_Definition:

Cropland.  Areas dominated by intensively managed crops.  These areas

typically require human activities for their maintenance.  This includes

areas used for the production of annual crops, such as corn, soybeans, wheat,

maize, vegetables, tobacco, cotton, etc.; perennial grasses for grazing; and

woody crops such as orchards and vineyards.  Crop vegetation accounts for

greater than 20 percent of total vegetation.  This class does not represent

natural grasslands used for light to moderate grazing.

LCCS codes: 10037, 10025, 21441, 21453.

Enumerated_Domain_Value_Definition_Source:

North American Atlas GIS Group

Attribute_Domain_Values:

Enumerated_Domain:

Enumerated_Domain_Value: 16

Enumerated_Domain_Value_Definition:

Barren lands.  Areas characterized by bare rock, gravel, sand, silt, clay, or

other earthen material, with little or no "green" vegetation present

regardless of its inherent ability to support life. Generally,  vegetation

accounts for less than 10 percent of total cover.  LCCS codes: 6001, 6004.

Enumerated_Domain_Value_Definition_Source:

North American Atlas GIS Group

Attribute_Domain_Values:

Enumerated_Domain:

Enumerated_Domain_Value: 17

Enumerated_Domain_Value_Definition:

Urban and built-up.  Areas that contain at least 30 percent or greater urban

constructed materials for human activities (cities, towns, transportation

etc.)  LCCS code: 5003.

Enumerated_Domain_Value_Definition_Source:

North American Atlas GIS Group

Attribute_Domain_Values:

Enumerated_Domain:

Enumerated_Domain_Value: 18

Enumerated_Domain_Value_Definition:

Water.  Areas of open water, generally with less than 25 percent cover of

non-water cover types.  This class refers to areas that are consistently

covered by water.  LCCS codes: 8001, 7001.

Enumerated_Domain_Value_Definition_Source:

North American Atlas GIS Group

Attribute_Domain_Values:

Enumerated_Domain:

Enumerated_Domain_Value: 19

Enumerated_Domain_Value_Definition:

Snow and ice.  Areas characterized by a perennial cover of ice and/or snow,

generally greater than 25 percent of total cover.  LCCS codes: 8005, 8008.

Enumerated_Domain_Value_Definition_Source:

North American Atlas GIS Group

Distribution_Information:

Distributor:

Contact_Information:

Contact_Organization_Primary:

Contact_Organization:

Commission for Environmental Cooperation

Contact_Address:

Address_Type: Mailing address

Address: 393, rue St-Jacques Ouest, Bureau 200

City: Montréal

State_or_Province: Québec

Country: Canada

Postal_Code: H2Y 1N9

Contact_Voice_Telephone: 1 514 350 4300

Contact_Facsimile_Telephone: 1 514 350 4314

Contact_Electronic_Mail_Address: info@cec.org

Distributor:

Contact_Information:

Contact_Organization_Primary:

Contact_Organization: Canada Centre for Remote Sensing

Contact_Position: Scientist

Contact_Address:

Address_Type: Mailing and physical address

Address: CCRS/NRCan

Address: 588 Booth Street

City: Ottawa

State_or_Province: ON

Postal_Code: K1A 0Y7

Country: Canada

Contact_Voice_Telephone: 1 613 947 1816

Contact_Facsimile_Telephone: 1 613 947 1383

Contact_Electronic_Mail_Address: Rasim.Latifovic@nrcan.gc.ca

Hours_of_Service: 0900 - 1700 ET, M - F (-5h EST/-4h EDT GMT)

Distributor:

Contact_Information:

Contact_Organization_Primary:

Contact_Organization: Instituto Nacional de Estadística y Geografía (INEGI)

Contact_Position: Director of Natural Resources and Environment Information

Contact_Address:

Address_Type: Mailing and physical address

Address: Av. Héroe de Nacozari Sur 2301. Fraccionamiento Jardines del Parque.

City: Aguascalientes

State_or_Province: Aguascalientes

Postal_Code: 20270

Country: Mexico

Contact_Voice_Telephone: 52 44 99 10 53 65

Contact_Electronic_Mail_Address: jose.ornelas@inegi.org.mx

Hours_of_Service: 0800 - 1600 (-6h GMT)

Distributor:

Contact_Information:

Contact_Organization_Primary:

Contact_Organization: U.S. Geological Survey

Contact_Person: Collin Homer

Contact_Address:

Address_Type: Mailing and physical address

Address: USGS, Earth Resources Observation and  Science (EROS) Center

Address: Mundt Federal Building

Address: 47914 252nd Street

City: Sioux Falls

State_or_Province: South Dakota

Postal_Code: 57198-0001

Country: USA

Contact_Voice_Telephone: 1 605 594 2714

Contact_Electronic_Mail_Address: homer@usgs.gov

Hours_of_Service: 0800 - 1600 CT, M - F (-6h CST/-5h CDT GMT)

Distribution_Liability:

Although these data have been processed successfully on computer systems at the CCRS,

INEGI, USGS, and the Commission for Environmental Cooperation, no warranty expressed

or implied is made by these agencies regarding the utility of the data on any other

system, nor shall the act of distribution constitute any such warranty.  Use of

trademarks or formats does not constitute endorsement by these agencies.  No

responsibility is assumed by the CCRS, INEGI, USGS, or the CEC in the use of these

data.

Standard_Order_Process:

Digital_Form:

Digital_Transfer_Information:

Format_Name: IMG

Digital_Transfer_Option:

Online_Option:

Computer_Contact_Information:

Network_Address:

Network_Resource_Name: http://www.cec.org/naatlas/

Network_Resource_Name: http://www.inegi.org.mx

Digital_Form:

Digital_Transfer_Information:

Format_Name: TIFF

Digital_Transfer_Option:

Online_Option:

Computer_Contact_Information:

Network_Address:

Network_Resource_Name: http://www.cec.org/naatlas/

Network_Resource_Name: http://www.inegi.org.mx

Digital_Form:

Digital_Transfer_Information:

Format_Name: MXD

Digital_Transfer_Option:

Online_Option:

Computer_Contact_Information:

Network_Address:

Network_Resource_Name: http://www.cec.org/naatlas

Digital_Form:

Digital_Transfer_Information:

Format_Name: GeoPDF

Digital_Transfer_Option:

Online_Option:

Computer_Contact_Information:

Network_Address:

Network_Resource_Name: http://www.cec.org/naatlas

Fees:

Gratuit-Free

Metadata_Reference_Information:

Metadata_Date: 20131025

Metadata_Contact:

Contact_Information:

Contact_Organization_Primary:

Contact_Organization: Commission for Environmental Cooperation

Contact_Address:

Address_Type: Mailing address

Address: 393, rue St-Jacques Ouest, Bureau 200

City: Montréal

State_or_Province: Québec

Country: Canada

Postal_Code: H2Y 1N9

Contact_Voice_Telephone: 1 514 350 4300

Contact_Facsimile_Telephone: 1 514 350 4314

Contact_Electronic_Mail_Address: info@cec.org

Metadata_Contact:

Contact_Information:

Contact_Organization_Primary:

Contact_Organization: Canada Centre for Remote Sensing

Contact_Person: Rasim Latifovic

Contact_Address:

Address_Type: Mailing address

Address: CCRS/NRCan

Address: 588 Booth Street

City: Ottawa

State_or_Province: ON

Postal_Code: K1A 0Y7

Country: Canada

Contact_Voice_Telephone: 1 613 947 1816

Contact_Facsimile_Telephone: 1 613 947 1383

Contact_Electronic_Mail_Address: Rasim.Latifovic@nrcan.gc.ca

Hours_of_Service: 0900 - 1700 ET, M - F (-5h EST/-4h EDT GMT)

Metadata_Contact:

Contact_Information:

Contact_Organization_Primary:

Contact_Organization: INEGI

Contact_Address:

Address_Type: Mailing address

Address: Av. Héroe de Nacozari Sur 2301. Fraccionamiento Jardines del Parque.

City: Aguascalientes

State_or_Province: Aguascalientes

Postal_Code: 20270

Country: Mexico

Contact_Voice_Telephone: 52 44 99 10 53 65

Contact_Electronic_Mail_Address: jose.ornelas@inegi.org.mx

Hours_of_Service: 0800 - 1600 (-6h GMT)

Metadata_Contact:

Contact_Information:

Contact_Organization_Primary:

Contact_Organization: U.S. Geological Survey

Contact_Person: Collin Homer

Contact_Address:

Address_Type: Mailing and physical address

Address: USGS, Earth Resources Observation and  Science (EROS) Center

Address: Mundt Federal Building

Address: 47914 252nd Street

City: Sioux Falls

State_or_Province: South Dakota

Postal_Code: 57198-0001

Country: USA

Contact_Voice_Telephone: 1 605 594 2714

Contact_Electronic_Mail_Address: homer@usgs.gov

Hours_of_Service: 0800 - 1600 CT, M - F (-6h CST/-5h CDT GMT)

Metadata_Standard_Name: FGDC Content Standard for Digital Geospatial Metadata

Metadata_Standard_Version: FGDC-STD-001-1998